ROGER ROV  |  DEPLOYMENT CHECKLIST

Date: ___________________    Site Location: _______________________________    Operator: ___________________

# SECTION 1  ·  SITE SETUP

| ✓ | TASK | INITIALS |
| --- | --- | --- |
| SHARED — All Personnel |  |  |
| ☐ | Lift manhole lid |  |
| ☐ | Start generator |  |
| ☐ | Unload equipment from the truck |  |
| ☐ | Confined Space Entry (CSE) — complete entry permits and lower ROGER into manhole |  |
| ☐ | Co-ordinate with CSE to drop and lift equipment for ROGER setup inside the manhole |  |
| PERSONNEL 1 — Deployment Case |  |  |
| Deployment Case |  |  |
| ☐ | Set up 2 laptops and connect to HDMI screens |  |
| ☐ | Connect chargers to both laptops |  |
| ☐ | Connect Ethernet cables |  |
| ☐ | Connect deployment case kettle cord (mains power) |  |
| ☐ | Connect controller to Surface PC |  |
| ☐ | Connect wireless mouse (if required) |  |
| ☐ | Connect encoder USB cable |  |
| Spool |  |  |
| ☐ | Connect Ethernet cable from deployment case to spool Ethernet port |  |
| ☐ | Connect spool power cable |  |
| ☐ | Connect spool USB cable for Icron Extender |  |
| Encoder |  |  |
| ☐ | Connect encoder power adapter |  |
| ☐ | Connect encoder USB interface cable |  |
| PERSONNEL 2 |  |  |
| ☐ | Set up tripod and winch |  |
| PERSONNEL 3 |  |  |
| ☐ | Unload toolboxes |  |
| ☐ | Set up encoder |  |
| ☐ | Set up power boards |  |
| ☐ | Safely manage and secure equipment around the site |  |

# SECTION 2  ·  SYSTEM STARTUP

| ✓ | TASK | INITIALS |
| --- | --- | --- |
| NETWORK & CONNECTIVITY |  |  |
| ☐ | Open ROGER UI on both surface laptops |  |
| ☐ | Verify ROGER network link is at 1 Gbps — If showing 100 Mbps: clean connectors with contact cleaner, then restart ROGER + spool Ethernet controller power |  |
| ☐ | Run connectivity speed test — must exceed 800 Mbps |  |
| ☐ | Sync ROGER clock to surface |  |
| SENSOR POWER-ON |  |  |
| ☐ | Turn on WBIB |  |
| ☐ | Turn on Livox LiDAR — enable USB Direct (Livox scanner) or LiDAR USB (BLK scanner) as appropriate |  |
| ☐ | Turn on Sonar |  |
| ☐ | Confirm 360° camera is on |  |
| SOFTWARE STACK STARTUP |  |  |
| ☐ | Start Robot Agent — confirm button turns red and status mirrors on second PC |  |
| ☐ | Start ROGER Stack on ROGER |  |
| ☐ | Start Camera Stack on ROGER |  |
| ☐ | Once ROGER stacks are running: start Surface Stack, then UAM Studio |  |
| ☐ | On streaming laptop: start Surface Video Stack |  |
| ☐ | On Windows laptop: open Sonar Surface software and confirm sonar connection |  |
| ☐ | Open UAM Studio → Tools → confirm Foxglove webpage loads correctly |  |

# SECTION 3  ·  PROJECT SETUP & INSPECTION RUN

| ✓ | TASK | INITIALS |
| --- | --- | --- |
| PRE-RUN CHECKS |  |  |
| ☐ | Confirm WBIB status shows "Device is Running" with all messages including BMS |  |
| ☐ | If WBIB is idle, use the single-service command to restart it |  |
| ☐ | In Job Control panel: enter project name and fill in all required parameters |  |
| ☐ | Position ROGER in the manhole and set encoder to zero (0) |  |
| STARTING THE RUN |  |  |
| ☐ | START Sonar recording |  |
| ☐ | Start and confirm the project in Job Control — status must progress from STARTING → RUNNING |  |
| DURING THE RUN |  |  |
| ☐ | Co-ordinate with team to drive ROGER and capture high-quality data |  |
| ☐ | Monitor image quality continuously (lens cleanliness, lighting, ROV angle) |  |
| ☐ | ⚠ If data quality is in doubt or any risk arises — stop and consult Craig. Craig has final authority to proceed or abort. |  |
| ENDING THE RUN |  |  |
| ☐ | STOP and confirm in Job Control — status must progress from STOPPING → STOPPED |  |
| ☐ | Stop Sonar recording |  |
| ☐ | ⚠ Drive back in coordination with manhole personnel and spool operator — keep tether taut at all times to avoid driving over it. Use reverse camera to verify tether tension. |  |

# SECTION 4  ·  RETRIEVAL & PACK DOWN

| ✓ | TASK | INITIALS |
| --- | --- | --- |
| DATA TRANSFER |  |  |
| ☐ | Once ROGER is stationary and on the winch: open Download Panel and start data transfer |  |
| RETRIEVAL |  |  |
| ☐ | Co-ordinate team to safely move ROGER out of the manhole |  |
| ☐ | Wash and clean ROGER |  |
| ☐ | Turn off ROGER |  |
| PACK DOWN |  |  |
| ☐ | Pack down all equipment and confirm everything is secured |  |
| ☐ | Check site — no tools, cables, or hardware left behind |  |
| ☐ | Close and replace manhole lid |  |

# SECTION 5  ·  THINGS TO LOOK OUT FOR

| ✓ | TASK | INITIALS |
| --- | --- | --- |
| TETHER & MECHANICAL |  |  |
| ☐ | While spooling in and out, inspect tether for cuts or abrasions — Note the chainage or mark the section for future repair if damage is found |  |
| ☐ | Calibrate hub motors if required |  |

# SECTION 6  ·  MAINTENANCE SCHEDULE

| ✓ | TASK | INITIALS |
| --- | --- | --- |
| DAILY MAINTENANCE |  |  |
| ☐ | Confirm all connectors and caps are tight and fully seated |  |
| ☐ | Inspect all cables for cuts or signs of damage |  |
| ☐ | Check for any degradation to lifting points and hardware |  |
| ☐ | Check for water ingress inside sensor housings and light lenses |  |
| ☐ | Inspect camera domes for scuff marks or scratches |  |
| ☐ | Confirm all seals are intact and screws are tight |  |
| WEEKLY MAINTENANCE |  |  |
| ☐ | Check hub motors for any signs of leaks |  |
| ☐ | Replace O-rings if worn or damaged and apply sealant as required |  |
| AS REQUIRED |  |  |
| ☐ | Calibrate hub motors and check for leaks |  |
| ☐ | Clean encoder PCB with contact cleaner |  |
| ☐ | Seal and replace O-rings as required |  |
| ☐ | Check tyre pressure on wheels |  |
| ☐ | Patch tether using potting compound as required |  |

ROGER Stack  ·  Roger Stack Project  ·  For internal field use only  ·  Version 1.0